# ShamanLink

- MicroSD Connector: https://www.digikey.com/en/products/detail/molex/0475710001/3262277 $1.03

- DualTVSProtection:https://www.digikey.com/en/products/detail/stmicroelectronics/USBLC6-2SC6/1040559?gclsrc=aw.ds&gad_source=1&gad_campaignid=9265913509&gbraid=0AAAAADrbLlg_nawixHFLelX4uDvWjshIf&gclid=Cj0KCQiA7rDMBhCjARIsAGDBuECDJjB5KYbxJCtVD6JINeEU68ExaBNAkUYsxQQHVYCZBCH4MHoYScYaAoSBEALw_wcB $0.36 x3

- Tactile Button: https://www.digikey.com/en/products/detail/c-k/PTS645SL43SMTR92-LFS/3861373 $0.36

- LDO: https://www.digikey.com/en/products/detail/diodes-incorporated/AP7361C-33Y5-13/5638323?gclsrc=aw.ds&gad_source=1&gad_campaignid=120565755&gbraid=0AAAAADrbLljItgRzsVnNO3-5qPvu9sOfC&gclid=Cj0KCQiA7rDMBhCjARIsAGDBuECiTWzj15IGrMk0ABcf4dpfvJElBhFm3Zu5wl6bDyvoj0KG5SSlQiwaAtZUEALw_wcB $0.50

- Priority MUX: https://www.ti.com/product/TPS2117 $1.15

- Crystal: https://www.digikey.com/en/products/detail/ecs-inc/ECS-160-20-46X/2676589 $0.37

- MCU: https://www.digikey.com/en/products/detail/nxp-usa-inc/LPC55S69JBD64K/10649974 $8.45

- USB-C Receptacle: https://www.digikey.com/en/products/detail/gct/USB4105-GF-A/11198441 $0.78

- DualTVSProtection:https://www.digikey.com/en/products/detail/stmicroelectronics/USBLC6-2SC6/1040559?gclsrc=aw.ds&gad_source=1&gad_campaignid=9265913509&gbraid=0AAAAADrbLlg_nawixHFLelX4uDvWjshIf&gclid=Cj0KCQiA7rDMBhCjARIsAGDBuECDJjB5KYbxJCtVD6JINeEU68ExaBNAkUYsxQQHVYCZBCH4MHoYScYaAoSBEALw_wcB $0.36 x1

- Polyfuse: 1206L075/13.2WR https://www.digikey.com/en/products/detail/littelfuse-inc/1206L075-13-2WR/2520739 $0.57

- TVSProtection: https://www.arrow.com/en/products/sm712-02htg/littelfuse?region=europe&utm_campaign=octopart_2022&utm_content=inv_listing&utm_currency=USD&utm_keyword=SM712-02HTG&utm_medium=aggregator&utm_source=octopart $0.59

- Connector: https://www.digikey.com/en/products/detail/jst-sales-america-inc/S6B-XH-A/1651034 $0.27

- Transceiver: https://www.ti.com/product/THVD1450/part-details/THVD1450D $2.78 SOIC option

- SWDTagConnect: https://www.tag-connect.com/product/tc2030-idc-nl $33.95

- MulticolorLED: https://www.digikey.com/en/products/detail/w%C3%BCrth-elektronik/150141M173100/4489960 $0.50

- Header Pins: https://www.digikey.com/en/products/detail/sullins-connector-solutions/PRPC036SAAN-RC/2775218?s=N4IgTCBcDaIMoEYAMCEFEDCBaAzANiwDkAREAXQF8g $0.85 for 3xmore than needed

- SMD Ferrite Beads: MFBW1V1608-601-R https://www.digikey.com/en/products/detail/eaton-electronics-division/MFBW1V1608-221-R/13161138 6x$0.10;

- 0805 ( >=0.125W rating; 1% tolerance; thick film) SMD Resistors: 5x10k; 1x22; 1x10k; 1x110k; 1x56k; 1x100k;  2x5.1k; 1x10k; 1x20k; 1x1M 2x10k; 4x33;3x10k 6x1k; 1x330; 2x10k; 2x0; 2x100k https://www.amazon.com/Chanzon-Resistor-Tolerance-Resistors-Certificated/dp/B08RYMY6XK/ref=sr_1_4?adgrpid=193180831344&dib=eyJ2IjoiMSJ9.hdE4wj5wOKZwJyNQjrScEMltfdXaixktsAvv8jhnDuwWZkWZLt1hNJFkn71d-DeU6mPxP2CveWxB7_wuiumg6EdKwrPRWs-FdKAt9WRfSEj9uM-mApPSEagLJgeFq8mUiRR8wviQpLZNOiFbHF9yee5M6cthRGbLI7byfqfrIpgNIsbkNTpHbhILC1yQ6QwNRH7Hl1KUZm8RteCrcAiNxMJ5DBtF8rwl62tuxl-2cWlwXaRbA51GOikfi1EuAqBr2-tfCp-7UyWoCvXk3qIOZP0yVZ-UPWbUhxk-qXFbdjc.ZUaxf6PQkXd2NkAQ6E3Xp067_E_N8Yecj5ZIQ7-mEGs&dib_tag=se&hvadid=779549316919&hvdev=c&hvexpln=0&hvlocphy=9051981&hvnetw=g&hvocijid=5061644063334794880--&hvqmt=e&hvrand=5061644063334794880&hvtargid=kwd-6397784619&hydadcr=2793_13539016_2118603&keywords=0805+resistor+kit&mcid=34a0300d3cbc3d94898e2f128c809d89&qid=1771602304&s=industrial&sr=1-4

- 1206 (>=0.25W rating) SMD Resistor: 1x120 https://www.amazon.com/dp/B08RYMYWG9/ref=sspa_dk_detail_0?psc=1&pd_rd_i=B08RYMYWG9&pd_rd_w=6IPYZ&content-id=amzn1.sym.7446a9d1-25fe-4460-b135-a60336bad2c9&pf_rd_p=7446a9d1-25fe-4460-b135-a60336bad2c9&pf_rd_r=PHXQW3PRZKSDFPY3CMVN&pd_rd_wg=zxbPn&pd_rd_r=e663cd61-4fa4-46c3-9f9b-5d4926fda25d&s=industrial&sp_csd=d2lkZ2V0TmFtZT1zcF9kZXRhaWw

- 0805 SMD Capacitors (>10V rating on all; dielectric matters): 1x0.1uF; 1x10uF; 2x10uF; 2x100nF; 2x4.7uF; 3x0.1uF; 2x33pF; 1x10nF; 1x 4.7nF; 10x0.1uF; 1x1uF; 2x10uF; 2x47pF; 1x100nF; 2x0.1uF; 1x10uF; 2x0.1uF;1x10uF 1x0.1uFhttps://www.amazon.com/0805-Capacitor-Capacitors-Assortment-Folder/dp/B0D6RW7CZ8/ref=sr_1_3?crid=61J9GME4ANZ1&dib=eyJ2IjoiMSJ9.5VV6G0vqhfKyEWe53xY9OSNuQDSSankDYtLxDmZtNQwUZdooIThQ57gWgXLhKSwsJMtbOrAP70ZtwQ2mFOSKRy-i8YQbCtGE6XHMB0mL5kD0hRtCC29op0IPzxTufyqajR-T_ldjx2w5UIlxBHbyxiRDAKMojSNQt_YwNV_Jc6r2wBEj1ppip55K7T8z98U5h0F7Le-_0Zo770PD2Z7sk3E-O8qaqbRHjARKfm1WGqY.w_c8Ytiq1pEFEQHh0p1WHaM_-fDXvVLhl4YCfT-EbAs&dib_tag=se&keywords=0805%2Bcapacitor%2Bkit&qid=1771603784&sprefix=0805%2Bcapacitor%2Bkit%2Caps%2C153&sr=8-3&th=1

- 1206 SMD Capacitors (>25V rating on all; dielectric matters) 1x22uF;  2x22uF; https://www.mouser.com/ProductDetail/Murata-Electronics/GRT31CC71E226ME13K?qs=xZFQr2mActe1lTnk7kyUaA%3D%3D

- SMD Inductors: CDRH2D11NP-4R7NC https://www.digikey.com/en/products/detail/sumida-america-inc/CDRH2D11NP-4R7NC/2620830 1x$0.75

- 0805 SMD LEDs: see the .csv file for links and datasheets 6->green,green,blue,yellow,yellow,red 6x$0.19 //make sure Vf < 3V on all LEDs and Vf > 1.5V for 2 LEDs in series with 33 ohm resistors

- Programmer: https://www.digikey.com/en/products/detail/nxp-usa-inc/MCU-LINK/13536147

- Solder materials: solder wire: https://www.amazon.com/Rosin-Solder-Electrical-Soldering-0-3mm/dp/B08M5XDKFG/ref=sr_1_1_sspa?crid=1NYOVKX57WBZ9&dib=eyJ2IjoiMSJ9.6LQQA_xYcvRoW92UoQ8CP7FfBfKUXjamj_Q2zOQNjL7zAd61HyTnH9QTovv0-pcg3B83UJNZrVvu6wcJMJhg4TTpuCZjAwNq6jWPvQBExQzGQKQAK-1__rtDpsiap46B3TMd82t02PkTkj-njZ1CJdKIfAh5ah-LIZKY0Q8tCEGfgrDQVJu5CXwkIV67QxlTVocIWshrVzhCbE_MIl38N1Q9w8Q64XkSfPt13qcF5icwDIcBr1UTdol7Vh_QvkDrowZtluR4fBYgpvQVgQ8BTgsr5OhmaKX4mCyGkL9gdoo.BYZt2tuKvZh2fo3FwyNduPWkvkpY0v3DuGnM5HT3aqA&dib_tag=se&keywords=solder%2B63%2F37%2Brosin%2Bcore%2B0.3mm&qid=1771605446&s=hi&sprefix=solder%2B63%2F37%2Brosin%2Bcore%2B0.3mm%2Ctools%2C176&sr=1-1-spons&sp_csd=d2lkZ2V0TmFtZT1zcF9hdGY&th=1 , solder paste: https://www.amazon.com/Solder-Paste-clean-63Sn-syringe/dp/B07B3VHPJ8/ref=sr_1_1?crid=VZ3VTZGBQTXR&dib=eyJ2IjoiMSJ9.Shvhc8zZiIIIGVb8jIXph4iHYJtvTTHyZetYzwCAvytVvwK3VjR6iz1ZjEkT7jAuWEFxeifSPkEn8NF9ow7dDptDhIn6gA8NHTusMX_Gw5CI6GKbEz6qz0esaHFHzR2p7YP9c_xMpz5psLbYyi1sYWzQuQR5XZHgxZE_kucmkwXKpFFok11gkLGUirh7uqR3xRPf5V5sfRISdYPJFDpmKxyJhegU_VsgFAHf5Y9EUNl5yP14CrdM7ogeDde2D6ZfnsMdjdzjPdVFjJrSyKAvhcy-ILpLgjR_6N1q8XRJJag.V0dYz_tsHme4y_Th30ackVIEwzToGfpXZFLN4O5IbYM&dib_tag=se&keywords=chipquik+5cc+solder+paste&qid=1771605661&sprefix=chipquik+5cc+solder+paste%2Caps%2C161&sr=8-1 , desoldering braid https://www.amazon.com/TOWOT-Desoldering-Disassemble-Electrical-Components/dp/B0943PDCC4/ref=sr_1_1_sspa?crid=U20OAGF3ZJ7J&dib=eyJ2IjoiMSJ9.HCqmZ3joz8N8kR-zg_zTJnhu_GOQmyP9ERsv_7l9uYSg86YQ2mO6llYm-sk7SPfe6m8qiO45ytzaG2FAnu6pYibbdcrXMm66Tc3OBiFru64ZKoQLUVAUVke3s6uf7_VhL8In0bRbQgeoxXwlO_xZCDoXQH0umwA_uGhrR0rJZX5JaWwoyONkE3S11NQRW1wcCtbDhz7tlWpq1LyIm1qWHTi8L0XSmxMahtqIhA7ZBB5I4Apa_5Xe4KsTwBMY4UX97-RHaogWXWp5-hCNxrPGRNLHqZwYcEoAljWGGmVJEQ0._fndnXaAKY2rTsQD3K250gZG9AhzcQgevfRxSPbWGBI&dib_tag=se&keywords=soldering%2Bbraid%2Bsize%2B2&qid=1771605838&sprefix=soldering%2Bbraid%2Bsize%2B2%2Caps%2C184&sr=8-1-spons&sp_csd=d2lkZ2V0TmFtZT1zcF9hdGY&th=1

# GeminiCHECKPowerBudgetShamanLink

### 1. The Power Supply Capability

- Source (USB-C): Standard USB 2.0 ports provide 500mA. USB 3.0 ports provide 900mA.

- Safety Fuse (Polyfuse 1206L075): This has a "Hold Current" of 750mA. This is your absolute input limit before the fuse starts acting like a resistor and cutting power.

- Voltage Regulator (LDO AP7361C):

  - Max Current: 1.0A (1000mA).

  - Constraint: While it can push 1A, it is limited by heat (see thermal check below).

Available Power Budget: Conservatively 500mA (limited by standard USB ports).

### 2. The Power Consumption (The Load)

We will calculate the "Worst Case" scenario (everything turned on and transmitting at once) for the 3.3V Rail.

| Component | State | Estimated Current (mA) | Notes |
| --- | --- | --- | --- |
| LPC55S69 MCU | Active Run (96MHz) | ~18 mA | Using internal DC-DC (implied by 4.7µH inductor). |
| MicroSD Card | Write Mode (Peak) | ~100 mA | This is the biggest variable. Some cards spike higher, but 100mA is a safe standard budget. |
| THVD1450 (RS485) | Transmitting | ~60 mA | Assumes driving a standard 54Ω termination load. |
| 6x LEDs | All On | ~8 mA | Your BOM lists 1kΩ resistors. $(3.3V - 2V) / 1000\Omega = 1.3mA$ per LED. |
| Multicolor LED | White (RGB On) | ~15 mA | Assumes ~5mA per channel. |
| TPS2117 Mux | On | ~0.1 mA | Negligible quiescent current. |
| Passive Leaks | TVS / Caps | ~0.1 mA | Negligible. |
| TOTAL (Peak) |  | ~201.2 mA |  |

Total Estimated Max Load: ~200 mA

### 3. Verification & Safety Margins

#### A. Current Capability [PASS]

- Demand: ~200mA

- Supply: 500mA (USB) / 1000mA (LDO) / 750mA (Fuse)

- Result: You are using roughly 40% of your available USB power. You have plenty of headroom for the MCU to boost its clock speed or for high-performance SD cards.

#### B. LDO Thermal Analysis [PASS]

This is critical. You are dropping 5V down to 3.3V. That extra voltage turns into heat.

- Voltage Drop: $5V - 3.3V = 1.7V$

- Current Load: $0.2A$ (200mA)

- Power Dissipated: $P = V \times I = 1.7 \times 0.2 = \mathbf{0.34 Watts}$

Your LDO (AP7361C) usually comes in SOT-89 or SOT-223 packages.

- Estimated Temp Rise: A standard SOT-89 package rises ~50°C per Watt.

- Calculation: $0.34W \times 50^\circ C/W \approx 17^\circ C$ rise.

- Result: If your room is 25°C, the LDO will run at roughly 42°C. This is warm to the touch but completely safe (well below the 125°C limit).

### 4. Special Component Notes

- Inductor (CDRH2D11NP-4R7NC): This 4.7µH inductor matches the requirements for the LPC55S69 Internal DC-DC Converter. This is good design; it keeps the MCU current consumption low.

- Resistors: The 1kΩ resistors you selected for the LEDs will make them quite dim (perfect for indicators that don't blind the user). If you want them brighter, you have plenty of power budget to swap them for 330Ω resistors later.

- Polyfuse: The 750mA hold current is perfect. It protects the USB port without tripping during normal SD card spikes.

### Conclusion

Your power budget is solid. You have approximately 300mA of spare capacity on the 3.3V rail.

| Component | State | Estimated Current (mA) |
| --- | --- | --- |
| LPC55S69 MCU | Active Run (96MHz) | ~18 mA |
| MicroSD Card | Write Mode (Peak) | ~100 mA |
| THVD1450 (RS485) | Transmitting | ~60 mA |
| 6x LEDs | All On | ~8 mA |
| Multicolor LED | White (RGB On) | ~15 mA |
| TPS2117 Mux | On | ~0.1 mA |
| Passive Leaks | TVS / Caps | ~0.1 mA |
| TOTAL (Peak) | Does not include powering a target board | ~201.2 mA |

Power Supply: ~500mA max from USB 2.0

Power Usage: ~40%

This is the "sanity check" phase, and it is arguably the most critical step before you order. You are looking to verify three things: Current Capability (will traces melt?), Voltage Drop (will the voltage be too low by the time it reaches the chip?), and Thermals (will the LDO burn up?).

Here is your step-by-step guide to verifying your power design using KiCad’s built-in tools and some simple math.

### Step 1: Create Your "Power Budget"

Before you measure anything, you need to know what you are measuring against. You cannot verify a trace if you don't know how many Amps are flowing through it.

- List your max currents:

  - USB Input (5V): What is the max current your board will draw? (likely < 500mA for a standard USB 2.0 device).

  - LDO Output (3.3V): How much does the MCU + LEDs + Peripherals consume? (LPC55S69 is usually < 20mA, but LEDs can add up).

  - Target Power: If you are powering an external target, how much current does it need?

Example Budget:

- Input 5V: 500mA (Max)

- Main 3.3V Rail: 200mA (MCU + LED + headroom)

### Step 2: Verify Trace Widths (KiCad Calculator)

You selected trace widths earlier (0.2mm, 0.4mm, 0.8mm). Now, let's scientifically prove they are big enough.

- Open KiCad Calculator: In the main KiCad project window, click the icon that looks like a calculator (or Tools -> Calculator Tools).

- Go to the "Track Width" Tab.

- Enter your parameters:

  - Current: Enter your max current (e.g., 0.5A for the USB line).

  - Temperature Rise: Standard is 10°C. (This means you are okay with the trace getting 10 degrees hotter than the room).

  - Trace Thickness: Standard 1oz copper is 35µm (0.035mm).

  - Trace Length: Enter a rough estimate (e.g., 20mm).

- Result: Look at the "External Layer Track Width".

  - Example: For 0.5A, KiCad might say you need only 0.15mm.

  - Your Design: You used 0.8mm for power. Verdict: PASS (with huge safety margin).

Rule of Thumb: A 0.3mm trace can easily handle 1 Amp on external layers. Your 0.8mm power traces are likely bulletproof.

### Step 3: Check Your Vias (The Bottleneck)

Vias are the chokepoints of power. A single standard via cannot handle as much current as a wide trace.

- In the Calculator: Switch to the "Via" tab (or look at the via section in Track Width).

- Enter your Via size:

  - Hole Diameter: 0.3mm (standard small via) or 0.4mm (power via).

  - Plating Thickness: Standard is 18µm - 25µm.

- Check the Current:

  - A single 0.3mm via can usually handle about 0.3A - 0.5A comfortably.

- The Fix: If you need to pass 1A, use multiple vias.

  - Rule: Always use at least two vias for any power connection (input, output, ground). This adds redundancy and lowers resistance.

  - Action: Go to your layout and verify that your USB Connector Ground pins and LDO Ground tab have multiple vias stitching them to the Ground Plane.

### Step 4: The "LDO Thermal" Check (Critical!)

This is the most common failure point. Your LDO drops 5V to 3.3V. That energy is turned into heat.

- Calculate Power Dissipation:
$$P = (V_{in} - V_{out}) \times I_{load}$$

  - $V_{in} = 5V$

  - $V_{out} = 3.3V$

  - $I_{load} = 0.2A$ (Example)

  - $P = (5 - 3.3) \times 0.2 = 1.7 \times 0.2 = \mathbf{0.34 Watts}$

- Check the Datasheet: Look for the "Thermal Resistance" ($R_{\theta JA}$) of your LDO package (e.g., SOT-23 or SOT-223).

  - SOT-23: A tiny package might heat up by $200^\circ C$ per Watt.

  - Calculation: $0.34W \times 200^\circ C/W = \mathbf{68^\circ C \text{ rise}}$.

  - Result: If your room is $25^\circ C$, the chip will be $93^\circ C$. That is HOT but safe (most silicon survives to $125^\circ C$).

- The Fix: If the calculation shows over $100^\circ C$:

  - Use a bigger package (SOT-223 instead of SOT-23).

  - Add more copper area around the tab (the heatsink).

  - Add "Thermal Vias" in the central pad to dump heat into the ground plane.

### Step 5: Verify Passive Components

- Resistors: Check any resistor carrying significant current (like LED current limiters).

  - $P = I^2 R$.

  - Example: 20mA through a 100-ohm resistor. $0.02^2 \times 100 = 0.04W$.

  - Rating: A standard 0402 resistor is rated for 0.063W (1/16W). This is close! An 0603 (0.1W) would be safer.

- Inductors/Ferrite Beads: Check the "Rated Current" in the datasheet.

  - If your USB input has a Ferrite Bead, ensure it is rated for at least 1.5x your expected current (e.g., rated for 1A if you expect 500mA).

### Summary Checklist

- [ ] Traces: Are power traces wider than the Calculator recommendation? (0.8mm is likely great).

- [ ] Vias: Do main power connections have at least 2 vias?

- [ ] LDO Heat: Did you calculate the temperature rise? Is it under $100^\circ C$?

- [ ] Passives: Are your resistors big enough (0603 vs 0402) for the power they dissipate?

Do you want to run the calculation for your specific LDO right now? If you give me the Part Number and your Estimated Current, I can tell you if it will overheat.

# MainTotemDemo

# MotorTotemDemo

# ModuleTotemDemo

# Scratchpad

## 1. Electrical & Physical Cleanup

- Run the Design Rules Checker (DRC): This is the most important step. Click the Bug icon in the top toolbar. It will catch any accidental shorts, traces that are too close together, or vias that didn't actually connect to the ground plane.

- Refill All Zones: Press the B key. This recalculates all copper pours. If you moved a via or trace, the copper plane might have a "hole" in it that needs to be filled.

- Check for "Islands": Look for any floating bits of copper that aren't connected to anything. KiCad’s DRC will usually flag these as "Unconnected items."

- Inspect Thermal Vias: Verify that your ground vias for U3 still show the GND net and are correctly merged into the bottom ground plane.

## 2. Silkscreen & Aesthetics

- Fix Overlapping Text: As we discussed, make sure C17, U3, and other labels aren't sitting on top of via holes or pads.

- Check Component Polarity: Ensure that "Pin 1" markers (dots or notches) are visible on the silkscreen so you don't solder the chips on backward.

- Add Board Information: It is always a good idea to add your name, the project title, the date, and a version number (e.g., "v1.0") to the F.Silk or B.Silk layer.

- Add Logos: If you want a "Certified Open Source" logo or a personal icon, add it now!

## 3. Hand-Soldering Optimizations

- Check Mask Openings: Use the 3D Viewer (Alt + 3) to confirm that the purple/pink areas (solder mask openings) are present on all pads you need to solder.

- Verify Clearance: Ensure there is enough physical room between components for your soldering iron tip to reach the pads without melting the plastic casing of a neighboring part.

## 4. Output Generation (The "Manufacturing" Files)

Once the DRC shows 0 Errors and 0 Warnings, it’s time to generate the files for the factory:

- Generate Gerbers: Go to File > Fabrication Outputs > Gerbers (.gbr). Select all copper layers, Silkscreen, Mask, and the Edge.Cuts (board outline) layer.

- Generate Drill Files: In the same menu, click Generate Drill Files. Use the default settings (Excellon format).

- Create a ZIP: Put all the generated .gbr and .drl files into a single ZIP folder. This is what you will upload to the manufacturer's website.

## 5. Final Visual Audit

- The "Paper Test": Some designers print the layout at 1:1 scale on a piece of paper and physically place their components on the paper to ensure the footprints are the correct size.

- Inspect the Bottom Layer: Flip the 3D model over. Ensure your ground plane is solid and your thermal vias are clearly visible for that "solder injection" trick.

6. Check that traces width all match their power amounts

7. Check your paper to make sure you have done everything

8. Read through all datasheets to make sure you followed all instructions and that the components you used all correctly match your power requirements.

9. Document out all of the PCB design process with specific steps so it’s easy to follow again and include everything you learned such as the powerpoint presentation tips on width for traces etc etc. all the Gemini stuff. Go through every prompt

10. Upload everything to GitHub and organize all of the ShamanLink documentation into the GitHub as well….. Email erosen

11. Final verifications of everything and order the PCB

12. Make sure all vias that need to be tented are tented